TACHEOMETRIC SURVEYING

Example 2 — Problem Statement & Complete Solution

Subject: Surveying  |  Topic: Tacheometry  |  Method: Subtense / Stadia

# 1. Problem Statement

To find the Reduced Level (RL) of station B, two observations are taken by a theodolite from station A — one to a Benchmark (BM) and the other to station B. The recorded observations are as follows:

| Inst. Station | Staff Station | Target | Vertical Angle | Staff Reading (m) | Remark |
| --- | --- | --- | --- | --- | --- |
| A | BM | Lower | −10°00′ | 0.655 | RL of BM = 510.500 m |
| A | BM | Upper | −7°00′ | 2.655 |  |
| A | B | Lower | −5°00′ | 1.250 |  |
| A | B | Upper | +4°00′ | 3.200 |  |

Required: (i) RL of station B     (ii) Horizontal distance between BM and station B

# 2. Given Data

- Instrument station: A

- RL of Benchmark (BM) = 510.500 m

- Observation 1 (A → BM): Lower vertical angle = −10°00′ (depression), Upper = −7°00′ (depression)

- Staff reading at BM: Lower cross-hair (h₁) = 0.655 m, Upper = 2.655 m

- Observation 2 (A → B): Lower vertical angle = −5°00′ (depression), Upper = +4°00′ (elevation)

- Staff reading at B: Lower cross-hair (h₂) = 1.250 m, Upper = 3.200 m

# 3. Principle & Formulae

## 3.1  Staff Intercept

The staff intercept S is the difference between the upper and lower cross-hair readings on the staff:

S = Upper cross-hair reading  −  Lower cross-hair reading

## 3.2  Horizontal Distance (Both angles on same side — both depression)

When both the lower and upper lines of sight are depressed (both negative angles α₁ and α₂):

S = D (tan α₁ − tan α₂)   →   D = S / (tan α₁ − tan α₂)

## 3.3  Horizontal Distance (Angles on opposite sides — one depression, one elevation)

When the lower cross-hair has a depression angle (−α) and the upper cross-hair has an elevation angle (+β), the angles are on opposite sides of the horizontal:

S = D (tan α + tan β)   →   D = S / (tan α + tan β)

## 3.4  Vertical Component

V = D × tan θ

where θ is the vertical angle to the lower cross-hair.

## 3.5  RL of Instrument Axis (from BM)

RL of instrument axis = RL of BM + h₁ + V₁

## 3.6  RL of Station B

RL of B = RL of instrument axis − V₂ − h₂

# 4. Step-by-Step Solution

| Step | Parameter | Calculation | Notes |
| --- | --- | --- | --- |
| 1 | Staff Intercept S1 | S1 = 2.655 − 0.655 = 2.000 m | Observation 1 (A→BM) |
| 2 | Horizontal Distance D1 | D1 = S1 / (tan10° − tan7°) = 37.31 m | D1 = 2.000 / 0.05362 |
| 3 | Vertical Component V1 | V1 = D1 × tan10° = 6.578 m | Depression angle used |
| 4 | RL of Instrument Axis | RL_inst = 510.500 + 0.655 + 6.578 = 517.733 m | RL(BM) + h1 + V1 |
| 5 | Staff Intercept S2 | S2 = 3.200 − 1.250 = 1.950 m | Observation 2 (A→B) |
| 6 | Horizontal Distance D2 | D2 = S2 / (tan5° + tan4°) = 12.396 m | Angles on opposite sides |
| 7 | Vertical Component V2 | V2 = D2 × tan5° = 1.085 m | Depression angle used |
| 8 | RL of Station B | RL(B) = 517.733 − 1.085 − 1.250 = 515.398 m | RL_inst − V2 − h2 |
| 9 | Distance BM to B | D1 + D2 = 37.31 + 12.396 = 49.706 m | Final answer |

## 4.1  First Observation (A → BM)

Both angles are on the same side (both depression). Staff intercept:

S₁ = 2.655 − 0.655 = 2.000 m

Horizontal distance D₁ (angles on same side — subtract tan values):

D₁ = 2.000 / (tan 10° − tan 7°)  =  2.000 / (0.17633 − 0.12278)  =  2.000 / 0.05355  =  37.31 m

Vertical component:

V₁ = 37.31 × tan 10°  =  37.31 × 0.17633  =  6.578 m

RL of instrument axis:

RL_inst = 510.500 + 0.655 + 6.578  =  517.733 m

## 4.2  Second Observation (A → B)

Angles are on OPPOSITE sides: lower = depression (−5°), upper = elevation (+4°). Add tan values:

S₂ = 3.200 − 1.250 = 1.950 m

D₂ = 1.950 / (tan 5° + tan 4°)  =  1.950 / (0.08749 + 0.06993)  =  1.950 / 0.15742  =  12.396 m

Vertical component:

V₂ = 12.396 × tan 5°  =  12.396 × 0.08749  =  1.085 m

RL of station B:

RL(B) = 517.733 − 1.085 − 1.250  =  515.398 m

# 5. Final Answers

(i)  RL of Station B  =  515.398 m

(ii) Distance BM to B  =  D₁ + D₂  =  37.31 + 12.396  =  49.706 m

# 6. Key Points to Remember

- When both sights are depressed (same side): use   D = S / (tan α₁ − tan α₂)

- When one sight is depressed and the other elevated (opposite sides): use   D = S / (tan α + tan β)

- The lower cross-hair reading is always taken as h (staff height at that station).

- V is always computed using the angle to the LOWER cross-hair and the calculated D.

- RL of instrument axis is found from the BM observation and stays constant throughout.

- RL of B is found by subtracting V₂ and h₂ from the RL of the instrument axis.

# 7. Interactive Python/Streamlit App

A fully interactive Python/Streamlit application is provided alongside this document. The app allows you to:

- Edit all input values (RL of BM, vertical angles, staff readings) via number input widgets

- See the tacheometric figure (Fig. E.11.2) update live with curved lines of sight, angle arcs, staves, and dimension lines

- View the complete step-by-step solution recalculated instantly for any input

- See final answers for RL of B and distance BM to B highlighted in colour

To run the app:

pip install streamlit matplotlib numpy

streamlit run app.py

— End of Document —